CardioFlex (Python)

A Python/PyQt5 re-implementation of Greenberg Lab's ‘CardioFlex.mlapp’.

MATLAB App Designer tool for analyzing tissue-stretch mechanics (EHT force/length recordings from an ASI muscle-mechanics rig).

Install

bash

pip install -r CardioFlex_Requirements.txt

python CardioFlex_PyGUI.py

Tested with Python 3.10+. No MATLAB installation or license required.

Files

- CardioFlex_Core.py — the analysis engine (file loading, trigger/step detection, peak/beat detection, contraction & relaxation kinetics, Poincaré/HRV, Frank-Starling curve fitting, group summary math). Pure NumPy/SciPy, no GUI dependency — importable and testable on its own.

- CardioFlex_PyGUI.py — the PyQt5 front end. Menu bar, "Analysis" and "Summary" tabs, and the tabbed figure window, structured to match the original MATLAB App Designer layout and callbacks one-for-one.

Execution

- Analysis tab → Select Files: choose one or more .dat recordings.

- Click a file in Selected Files to load it — this shows the overall length/force trace in a pop-out window and detects (or lets you manually assign, via the menu) the 8 stretch windows.

- Click a stretch in Select Stretch # to Plot/Analyze — the main plot shows the stimulus-labeled force trace for that window; a dialog then asks which beat numbers to use for kinetics (defaulting to the most recent N, where N is the Number of Beats spinner).

- Repeat for each stretch, then Combine Selected Stretches to get the Frank-Starling curves and pooled Poincaré/HRV plot.

- Save Data and Figures writes a tab-delimited results table plus all ten .png/.eps figures into <filename>_Output/.

- Summary tab: after analyzing multiple files (via the workflow above, using Save Data and Figures to produce each file's (..._output.tsv), use Group 1/2/3 to select that group's output files, combine them into a per-group .xlsx workbook, and plot length-dependence curves (with quadratic fits) across the group.